HUD System Operation Manual

A. Data connector  B. Wheel Button  C. Sensitization Hole  D. Display Screen

-1-

Installation Process

Step 1: Keep the car stall and plug the OBD line into the car's OBD port.

Step 2: Start the car.

Step 3: Plug the other side of the OBD line into HUD.

Button Functions

Normal mode:

Scroll down: switch the display

(Speed, Water temperature and voltage)

Scroll up: Overspeed alarm Settings （from 40 to 180KM/H or MPH）

Pressed inward and hold: Start setting mode

Setting mode:

Scroll down: Add

Scroll up: Subtract

Pressed inward: Save

Pressed inward and hold: Exit setting

-2-

Screen Function Introduction

- Water temperature Alarm     B. Digital Display Area

- Voltage Alarm                 D. Unit Symbol

1 Speed, Water temperature and voltage

2 Overspeed alarm, High temperature alarm, Low voltage alarm

3 Display mileage after flameout (Because the biggest display value is 199,so when the mileage value will count again after more than 199,for example the actual mileage value is 250km,is shown as 250-199=51)

-3-

Setting mode Introduction

Pressed inward and hold to start setting mode, after entering the setting mode will  display the version number and then enter the settings, at this time can be set up through the wheel up and down key to select other item, pressed inward to enter setting values, through the wheel up and down key to change setting values, pressed inward to return to set items, pressed inward and hold to save and exit the setting mode

Speed Unit (1-2):The default value is 1-KM/H, 2 is MPH.

Protocol selection (0-11):The default value is 0,0:Auto-Select.

Select car category: The default value is 1,1:Normal. 2:Hybrid or i-Stop.

Speed calibration (80-120):The default value is 108,Please adjust according

Water temperature alarm value(90-130):The default value is 110,Please adjust according to the actual car status.)

-4-

Brightness(0-7):The default value is 0,0-Automatic Brightness,1-Lowest  Brightness,7-Highest Brightness.

Shift reminder setting (8 ~ 70): the default value is 50, which means 5000 shift reminder. If the vehicle speed exceeds 60 km/h, it will not be reminded

Product Size and Electrical

Specifications                   Accessories

L：               80Mm                           HUD Module        x1

W：               45Mm                          Certificate           x1

H：               13Mm                           OBD line            x1

Voltage：        OBDII 12V                          Operation          x1

Maximum Current      150Ma                        3M                x1

Standby Current      <20Ma                         lens hood           x1

Sleep Standby Current   <6Ma                        reflector film      x1

Operating temperature： -40℃~85℃

-5-

Important Notice

Please do not operate in the flameout state, otherwise it may cause the head up display to stay on and consume power!!!

1．If the car will be parked for many days, please put out the OBD line

2．The imagers in this manual are for reference only

3．The vehicle must be in a parked position before operating the HUD

-6-