Brief User Guide of Software Upgrade

Step 1: please check the correct language version of V60 HUD at your hand.

Step 2: Long press device LEFT button and hold, connect the USB cable to computer, you will see device interface is black with white fonts (loose your hand to press the button)

Step 3: Check your computer system and install the relative driver.

Step 4: Open the program “DfuSeDemo.exe” as below, and click “choose” to choose “V60-EN-ASHdfu.dfu” file, then click “upgrade”.

Step 5: once upgrade successfully, you can see below interface and device is on automatically:

Step 6: Reboot the machine as below steps:

If the screen is colorful after upgrade software successfully, please don't unplug the HUD from computer, need to reboot the machine: press the left button for 8 times; later press the right button for 8 times; finally press the middle "OK" button one time, then the machine will reboot. (	If the screen is normal, then ignore this step).

Step 7: Connect the 	HUD to your car to check again.